# Microbial Culture Shakers

Contents

Purpose	1

Scope	1

Responsibilities	1

Supervisors	1

Workers	2

Equipment Needed	2

Before You Begin	2

Running the Shaker	2

Retrieving Your Samples	2

# Purpose

<<<< This SOP is left in DOCX format so that you may edit it for your own laboratory>>>

The purpose of this SOP is to lay out the responsibilities, equipment and procedures required for use and maintenance of microbial culture shakers.

# Scope

This SOP applies to all persons prescribing the use of and requiring to use microbial culture shakers.

# Responsibilities

## Supervisors

Supervisors are responsible for:

- Reviewing this SOP on a regular basis. Review is to consider and mitigate the risks of spill, loss of containment and exposure or other harm. Refer to Performing Risk Assessments SOP.

- Ensuring all microbial culture shakers in their area are clean and in good working order.

- Ensure all shakers are maintained regularly.

- Ensuring all shakers are labelled with the correct contact information for the owner of the shaker should service or maintenance be required.

- Ensuring service calls are placed when shakers require repair.

- Ensuring that all workers under their supervision are trained on and are proficient in performing the steps of this SOP.

## Workers

Workers are responsible for:

- Following this SOP as approved by their supervisor to use and maintain microbial culture shakers.

- Reporting any damage or malfunctioning immediately to their supervisor.

# Equipment Needed

- User manual for shaker

- Appropriate tools for changing out holders

- Spray bottle of disinfectant

- Paper towels

# Before You Begin

- Read the user manual to be able to perform the steps of this SOP.

- Ensure electrical cord is plugged in securely.

- Ensure your tubes or flasks are not overfilled (recommended 500ml or less when using a 2L flask and 2ml or less when using bacteriological tubes)

- Ensure your tubes or flasks are closed to prevent splashes if RG1.

- Ensure your tubes or flasks are closed with HEPA filter, vented caps if RG2.

- Ensure your tubes or flasks are labelled with the following:

  - Your name

  - Your contact #

  - Risk group level of the organism (RG1 or RG2)

- Load your flasks or tubes into the shaker.

- Make sure the shaker platform is balanced as much as possible.

# Running the Shaker

- Turn on the shaker.

- Set the appropriate temperature.

- Increase the RPM’s slowly to research the desired speed.

- Watch the shaker to ensure flasks are secure and the platform is balanced.

# Retrieving Your Samples

- Decrease the RPM’s until the platform comes to a complete stop.

- Inspect the area to determine if a spill has occurred.

  - Turn the shaker off, sign the equipment with a spill. Notify all owners of tubes/flasks within the shaker. Leave closed for 30 minutes.

  - After 30 minutes open the shaker retrieve any broken glass with tongs into a biohazard sharps container.

  - Wipe down all intact samples with disinfectant and remove. The owner of those samples is responsible for their disposition.

  - Lay down paper towels to absorb puddles of liquids.

  - Add disinfectant to the paper towels and let sit for minimum contact time.

  - Mist the remainder of the surfaces inside the shaker.

  - Retrieve all paper towels into solid biohazardous waste.

  - Saturate paper towels in disinfectant and wet-wipe the inside surfaces of the shaker twice.

  - Retrieve all paper towels into solid biohazardous waste.

  - If bleach or other corrosive chemical is used as a disinfectant, wet-wipe the interior surfaces with paper towels saturated with tap water.

- Retrieve your samples.

- Close the lid and bring back to appropriate RPMs if there are other samples present.

- Decrease RPMs to zero if there are no other samples present.

- Turn off shaker if there are no other samples present.