# Use of Sharps

Contents

Purpose	1

Scope	1

Responsibilities	1

Supervisors	1

Workers	2

Equipment Needed	2

Use of Microtome Blades	2

Use of Scalpels	2

Use of Needles	3

Training	3

# Purpose

<<<< This SOP is left in DOCX format so that you may edit it for your own laboratory>>>

The purpose of this SOP is to lay out the responsibilities, equipment and procedures required for using sharps.

# Scope

This SOP applies to all persons prescribing and requiring to use sharps.

# Responsibilities

## Supervisors

Supervisors are responsible for:

- Reviewing this SOP on a regular basis. Review is to consider and mitigate the risks of spill, loss of containment and exposure or other harm. Refer to Performing Risk Assessments SOP.

- Ensure approved sharps containers are provided.

- Ensuring that all workers under their supervision are trained on and are proficient in performing the steps of this SOP.

## Workers

Workers are responsible for:

- Following this SOP as approved by their supervisor

- Reporting any sharps-related incidents to their supervisor

# Equipment Needed

- Sharps disposal container

# Use of Microtome Blades

- Always place the blade-guard over the blade.

- Always lock the wheel when not sectioning.

- Always back up the chuck when changing blocks. NEVER place your finger on or around the block when the wheel is unlocked.

- Always dispose of a blade in the blade disposal section of the blade dispenser or appropriate sharps container immediately upon removal from the microtome.

# Use of Scalpels

- Use disposable scalpels whenever possible.

- Use forceps to attach and remove a scalpel blade from a scalpel handle

- Always cut away from your holding hands.

- Always dispose of a scalpel in an appropriate sharps container immediately upon removal from the scalpel handle.

# Use of Needles

- Use safety engineered needles whenever possible.

- Never bend or alter needles.

- As much as possible, avoid recapping needles.

  - If recapping, use a safe, one-handed method of recapping SOP or a recapping device.

- Always discard needles immediately after use in an approved sharps container.

- Never use needles as darts or thumbtacks.

- <<NEW>> Dispose of unused, wrapped, capped needles as-is into sharps containers if required.

- <<NEW>> Needles should NOT contain liquids. Eject liquids into appropriate waste container and dispose accordingly.

- <<NEW>> Needles used with cytotoxic drugs are to be discarded into sharps containers labelled with CYTOTOXIC WASTE label

- https://www.cdc.gov/niosh/docs/2010-167/pdfs/2010-167.pdf

# Training

- Each lab member is to read, agree and sign off on this SOP.

- Upon revision or re-write, this SOP is to be re-circulated and re-signed by all lab members.