# Laboratory Layout

Contents

Purpose	1

Scope	1

Responsibilities	1

Supervisors	1

Workers	2

Equipment Needed	2

Types of Areas	2

Recommendations	2

Creation of the Layout	3

Training	3

# Purpose

<<<< This SOP is left in DOCX format so that you may edit it for your own laboratory>>>

The purpose of this SOP is to lay out the responsibilities and procedures required for working in a laboratory with specific areas dedicated for specific uses, when the option of having separate rooms is not available, in order to prevent cross-contamination.

# Scope

This SOP applies to all persons working in a laboratory with multiple uses and their Supervisors.

# Responsibilities

## Supervisors

Supervisors are responsible for:

- Reviewing this SOP on a regular basis. Review is to consider and mitigate the risks of spill, loss of containment, cross-contamination and exposure or other harm. Refer to Performing Risk Assessments SOP.

- Ensuring all workers have access to facilities to store personal belongings.

- Ensuring the layout of dedicated areas is appropriate for the work undertaken in those areas.

- Ensuring the assignment of dedicated paperwork spaces and personal storage spaces prevents the risk of cross contamination of paperwork or personal items.

- Ensuring that all workers under their supervision are trained on and are proficient in performing the steps of this SOP.

## Workers

Workers are responsible for:

- Following this SOP as approved by their supervisor

- Report any new configurations of space that are required prior to initiating the reconfiguration.

# Equipment Needed

- Layout of all laboratory areas

- Signage indicating layout and dedicated areas

# Types of Areas

- Containment Level 2 Laboratory – A room or suite of rooms where risk group 2 and risk group 1 biohazards are being stored and handled.

- Containment Level 1 Laboratory – A room or suite of rooms where risk group 1 biohazards are stored and handled.

- Level 2 work area – a lab work area where risk group 2 and risk group 1 biohazards are being handled. Can only be located in a Containment Level 2 laboratory.

- Level 1 work area – a lab work area where risk group 1 biohazards are being handled. Can be located in a Containment Level 2 laboratory or a containment Level 1 laboratory.

- Non-biohazard work area – a work area where lab work is being done, such as preparing chemicals, where no biohazards are present. Can be located in a Containment Level 2 laboratory or a containment Level 1 laboratory.

- Personal storage area – a dedicated area for storing personal items, for example a set of hooks for street clothes or a set of cubby’s for storing bags and purses

- Dedicated Paperwork Area or Desk Area – a dedicated area for writing reports and doing computer work, usually a series of desks or benches where students sit inside the laboratory.

- Lab storage area – dedicated area for storage of supplies and equipment where no wet-work is to be carried out.

# Recommendations

- Dedicated paperwork areas or desk areas and personal storage areas be buffered by non-work areas i.e. desks beside filing cabinets or supply shelving rather than beside a wet-work area.

- Cardboard biohazard boxes be kept away from areas where contamination can occur. Pre-bagging level 1 waste in clear bags is preferred.

- Cardboard storage boxes be kept away from wet-work areas.

- Dedicated containers for each of biohazardous waste , regular laboratory waste and desk area waste.

- Level 1 and Level 2 work areas be compressed as much as possible.

- Place chairs made of non-absorbent materials in Level 1 and level 2 work areas and leave chairs made of absorbent materials in desk areas or non-biohazard areas.

# Creation of the Layout

- Building maps can be found at http://www.mcmaster.ca/uts/maps/ if you are non-hospital located.

- HSC map screencaps of your area can be provided by the Biosafety Office. There are no maps available for other hospital-hosted locations.

- Draw in location of benching, equipment and desks.

- Draw boundaries for each separate type of work area.

- Label each separate type of work area.

- Attach maps to this SOP.

# Procedures

- Dedicated paperwork and desk areas to be free of biological contamination.

- PPE to be doffed prior to entry into the dedicated paperwork and desk areas.

- Hands to be washed prior to entry into the dedicated paperwork and desk areas.

- Items to be brought into the dedicated paperwork and desk areas to be decontaminated.

- No hazardous materials shall be discarded in waste receptacles in the dedicated paperwork and desk areas.

# Temporary Dedicated Paperwork and Desk areas

- All procedures are to be followed.

- Signage may be posted at the beginning of the period where temporary space is required.

- Signage to be removed at the end of the period where temporary space is required.

# Training

- Each lab member is to read, agree and sign off on this SOP.

- Upon revision or re-write, this SOP is to be re-circulated and re-signed by all lab members.

https://www.ontario.ca/document/management-biomedical-waste-ontario