# Use of Vortex

Contents

Purpose	1

Scope	1

Responsibilities	1

Supervisors	1

Workers	2

Equipment Needed	2

Installation	2

Continuous Operation	2

Intermittent Operation	2

Vortexing of Biohazardous Materials	3

Training	3

# Purpose

<<<< This SOP is left in DOCX format so that you may edit it for your own laboratory and your own make and model of vortex mixer>>>

The purpose of this SOP is to lay out the responsibilities, equipment and procedures required for the use of a laboratory vortex.

# Scope

This SOP applies to all persons prescribing and requiring to use a laboratory vortex.

# Responsibilities

## Supervisors

Supervisors are responsible for:

- Reviewing this SOP on a regular basis. Review is to consider and mitigate the risks of spill, loss of containment and exposure or other harm. Refer to Performing Risk Assessments SOP.

- Ensuring the equipment is in good working order.

- Ensuring that all workers under their supervision are trained on and are proficient in performing the steps of this SOP.

## Workers

Workers are responsible for:

- Following this SOP as approved by their supervisor

- Reporting any broken equipment immediately to their supervisor

# Equipment Needed

- Vortex mixer

# Installation

- Place on a level surface and plug cord into mains supply.

- Place desired attachment onto the shaft and rotate until the ‘flats’ align.

- Press the attachment down firmly until it snaps into place.

- Grasp and pull straight up the installed attachment until it is released from the shaft if another type of attachment needs to be used.

- Always clean the attachment after use.

- Always use the appropriate attachment for the vessel.

# Continuous Operation

- Push switch to the ‘ON’ position.

- Turn speed control knob to setting No.3 on the dial.

- Observe that the attachment is immediately set in motion.

- Place the sample tube on top of the attachment and keep steady.

- Vary the speed on the dial and/or the angle of contact and pressure against the mixing attachment to achieve the desired mix.

- Return switch to the ‘OFF’ position once finished.

# Intermittent Operation

- Push switch to the ‘TOUCH’ position.

- Turn speed control knob to setting No.3 on the dial.

- Touch the sample tube with the attachment to observe motion and keep steady.

- Vary the speed on the dial and/or the angle of contact and pressure against the mixing attachment to achieve the desired mix.

- Return switch to the ‘OFF’ position once finished.

# Vortexing of Biohazardous Materials

- Use and appropriate vessel for the sample.

- Ensure the cap or lid is on tight before vortexing.

- Grasp the tube with a paper towel, and vortex the tube perpendicularly

- Wait 2-5 minutes before opening the vessel after vortexing.

- If risk group 2, open the vessel in a biological safety cabinet, or bring the vortex to the biological safety cabinet.

- Screw-cap, o-ringed tubes are preferable over snap-cap tubes.

# Training

- Each lab member is to read, agree and sign off on this SOP.

- Upon revision or re-write, this SOP is to be re-circulated and re-signed by all lab members.